WinNwt4 – Windows Installation

- Unzip file and install “winnwt_4_09” application.

- Connect spectrum analyzer to USB port on your computer

- Go to Device manager and identify com port for spectrum analyzer

- Launch WiNwt4.  Under settings, select Options.

- In Serial interface box, insure that proper port is selected

- In Frequency Multiply, insure that “10” is selected

- Connect source to input of spectrum analyzer. Set frequency of source. In main dialog box, select “Continuous”.  Spectrum of signal should appear on plot.